15.211: Minimum Setback Distances

(1) All systems must conform to the minimum setback distance for septic tanks and soil absorption systems, including reserve area, measured in feet and as set forth below. Where more than one setback applies, all setback requirements shall be satisfied.

Septic Tank 		       Soil Absorption System

Property Line 				10 				10

Cellar Wall or

Swimming Pool (inground) 		10 				20

Slab Foundation 				10 				10

Water Supply Line (pressure) 		10[1] 				10[1]

Surface Waters (except wetlands) 		25 				50

Bordering Vegetated Wetland (BVW),

Salt Marshes, Inland and Coastal Banks	 25 				50

Surface Water Supply -

Reservoirs and Impoundments 		400 				400

Tributaries to Surface

Water Supplies				200 				200

Wetlands bordering Surface Water Supply

or Tributary thereto 			100 				100

Certified Vernal Pools 			50 				100[2]

Private Water Supply Well or Suction Line 	50 				100

Public Water Supply Well 			(2) 				(2)

Irrigation Well 				10 				25

Open, Surface or Subsurface Drains which

discharge to Surface Water Supplies or

tributaries thereto 			50 				100

Other Open, Surface or Subsurface Drains

(excluding foundation drains) which

intercept seasonal high groundwater

table [3] 				25 				50

Other Open, Surface or Subsurface Drains

(excluding foundation drains) 		5 				10

Leaching Catch Basins &

Dry Wells	 			10 				25

Downhill Slope				 not applicable 			15[4]

[1] Disposal facilities shall also be at least 18 inches below water supply lines. Wherever

sewer lines must cross water supply lines, both pipes shall be constructed of class 150

pressure pipe and shall be pressure tested to assure watertightness.

[2] The required setback shall be 50 feet where the applicant has provided hydrogeologic

data acceptable to the approving authority demonstrating that the location of the soil

absorption system is hydraulically downgradient of the vernal pool. Surface topography alone

is not determinative.

[3] Surface or subsurface drains which will regularly or periodically intercept the seasonal

high groundwater table and carry that groundwater away from an area must meet the specified

setbacks.

[4] The setback distance shall be measured from a naturally-occurring downhill slope which

is not steeper than 3:1 (horizontal:vertical). A minimum 15 foot horizontal separation

distance shall be provided between the top of the peastone in the soil absorption system and

the adjacent downhill slope. For a system located in an area with any adjacent naturally

occurring downhill slope steeper than 3:1, slope stabilization shall be provided in accordance

with best engineering practice which may include construction of a concrete retaining wall

constructed in accordance with 310 CMR 15.255(2).